Pipeline Pressurizing Test Documents

Common description:

Table 1 shows the specification of the pipeline used.

| Contents | Value |
| --- | --- |
| Thickness | t > 4.85mm (-12.5% to INF.) |
| Length | 2m |
| Size | 150A |
| Notch size | 0.1mm x 10mm x 2.7mm (width x length x depth) |
| Notch center position | 295mm (from a side) |
| Metal | SPP(KSD3507) C(-) MIN(-) P(0.040%) S(0.040%) |
| Design pressure | 25Bar |
| Out diameter | 6’’ (165.2mm) |
| Inner diameter | 155.5mm (not exactly) |

Fig. 1 Pipeline diagram

In this experiment, six pipes, one plain pipe and five notched pipes were used. The pipe was pressurized to 0 ~ 200 Bar by nine stages of pressurization. The first pressure is 0 Bar to check the background noise, the second is 50% of the design pressure, the third is 65%, the fourth is 80%, the fifth is 100%. And in 4 stages (50, 100, 150, 200) further pressurize. Data collection times are 30, 10, 10, 10, 30, 5, 5, 5, 5 min at each stage. Pressure data is stored in each folder "pressure.mat".

Sampling rate : 1M Hz

Sensor Type : R15I-AST

Sensor position :

Fig. 2 Position of sensor mounter and points of pencil leak break
(20mm spacing for each position)